GUIDANCE ON COMPLETION OF SECTIONS 1 AND 3 OF THE IUCLID 5 DOSSIER:

IRON [EINECS number 231-096-4, CAS number 7439-89-6]

as Iron powders

INTRODUCTION	2

1. GENERAL INFORMATION	3

1.1	IDENTIFICATION	3

1.2	COMPOSITION	6

1.3	IDENTIFIERS	11

1.4	ANALYTICAL INFORMATION	12

1.5	JOINT SUBMISSION	13

1.6	SPONSORS	14

1.7	SUPPLIERS	15

1.8	RECIPIENTS	16

3. MANUFACTURE, USE AND EXPOSURE	17

3.1	TECHNOLOGICAL PROCESS	17

3.2	ESTIMATED QUANTITIES	20

3.3	SITES	21

3.4	INFORMATION ON MIXTURES	22

3.5	LIFE CYCLE DESCRIPTION	23

3.6	USES ADVISED AGAINST	25

3.7	Exposure Scenarios, exposure and risk assessment	25

3.7.1	Exposure scenarios and local assessment	25

3.7.2	Environmental assessment for aggregated sources	25

3.7.3	Generic exposure potential	26

3.8	BIOCIDAL INFORMATION	27

RECOMMENDATIONS CONCERNING SUBMISSION	27

# INTRODUCTION

This document details the information submitted by the Lead Registrant for Iron [with respect to iron powders] and is intended as a guide to member registrants for completion of the necessary fields of sections 1 and 3 of their IUCLID 5.4 dossiers for Iron.

Note however that the text of REACH Regulation is the only authentic legal reference and the information contained in this document does not constitute legal advice. It is therefore recommended that member registrants should read all relevant ECHA Guidance documents, for example Practical Guide 9: How to do a registration as a member of a joint submission (August 2010).

Member registrants should make themselves aware of all new updates of the IUCLID software and its plug-ins (IUCLID installation kit).

This document contains two types of information:

- that which will be common to all dossiers which are part of the Joint Submission for Iron - the cells for which in this document are highlighted in orange;

- that which is particular to your company - the cells for which in this document are highlighted in blue;

- Cells highlighted in grey relate to headings only and have no content.

In order to input data to fields, please click on the EDIT button or select Control-E.

Do not forget to save data entered by clicking on the save button.

Do not forget to check your substance and dossier files with the IUCLID 5.4 Technical Completeness Check (TCC) plug-in tool [don’t forget to update this plug-in].

If confidentiality is required, the registration fee will be more expensive and a justification has to be provided!   Note that ECHA has issued in July 2012 a guidance document on confidentiality claims - this can be downloaded from the ECHA website - Data submission manuals or from the Library page of the Iron Platform website.

A fee calculator plug-in is available. This plug-in assists Legal Entities in calculating fees associated to REACH or CLP dossiers.

Information on the creation of a new substance is available on the Iron Platform website in the SLIDES FROM IUCLID AND REACH-IT WEBINAR 08/07/2010 [slides 14-20]. Member registrants will have to import all IUCLID files [reference substances and the file containing the uses] provided by the Iron Platform before creating their substance files. A guidance document “How to import an i5z file into IUCLID 5.2” is available on the Iron Platform website [also applicable for IUCLID version 5.4].

An IUCLID 5.4 dissemination plug-in is available. This allows a registrant to preview or simulate the information from its registration dossier that ECHA will make available via the internet. You can find more information on disseminated data in the ECHA Data Submission Manual, Part 15 - Dissemination (July 2012) and its Technical annex for IUCLID section 1, 2, 3 (July 2012) - these documents are also available on the Library page of the Iron Platform website.

Note: The new version of IUCLID does not have an impact on TCC requirements for the moment. New fields related to the CSR information will not be subject to TCC until 2014 (see ECHA Q&A on IUCLID 5.4 (April 2012) for more information - also on the Library page of the Iron Platform website). Members are free to fill in these new fields.

# 1. GENERAL INFORMATION

## 1.1	IDENTIFICATION

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Substance identification | Heading only |  |
| Chemical name | Iron |  |
| Public name |  | Enter the generic name by which you describe your substance, e.g. atomised iron powders, carbonyl iron powders, etc. |
| Legal entity or third party flags: |  | Click on the flag if you want to assign confidentiality and programme restrictions. |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Role in the supply chain |  | Choose your role and tick the appropriate box. Note: - If “manufacturer is selected, a production site is needed in section 3.3 - “downstream user” cannot be selected if the submission covers only intermediates - If “downstream user” is selected, “substance in article” must be ticked in section 3.4 - “only representative” cannot be selected together with “manufacturer” or “importer” |
| Role flags |  | Click on the flag if you want to assign confidentiality and programme restriction |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Reference substance | Heading only |  |
| Reference substance flag |  | Click on the flag if you want to assign confidentiality and programme restrictions. |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Reference substance | iron / iron / iron / 7439-89-6 | To locate the reference substance from the IUCLID data base, click on this icon [see red arrow in screenshot above]. Select your substance from the database by typing in the name, EC or CAS number, click SEARCH, select the substance name and click Assign [see screenshot below]. Two problems may arise: • If no entry is found, you have first to import the substance from the EC inventory to the reference substance inventory. • If an entry is found but inactive, right mouse click and set to “active reference substance.” In order to simplify matters, the Iron Platform will provide reference substance files which member registrants can import into their IUCLID dossiers. |
| EC number / name |  | This information is automatically provided when the reference substance is assigned. |
| CAS number |  | This information is automatically provided when the reference substance is assigned. |
| IUPAC name |  | This information is automatically provided when the reference substance is assigned. |
| Type of substance: | Heading only |  |
| Composition | mono constituent substance |  |
| Origin | inorganic |  |
| Trade names | Heading only |  |
| Trade names flag: |  | Click on the flag if you want to assign confidentiality and programme restriction |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Name |  | If you have trade name[s] for your substance, add it/them here |
| Contact person |  | These fields are for your own company information. The details entered should correspond with the information contained in REACH-IT. Several contacts can be provided. Click on to add one. |

## 1.2	COMPOSITION

Note: The following table summarises the information on composition to be added in Chapter 1.2 of IUCLID. The degree of purity and additives sections are common for all types of iron powders including Carbonyl Iron Powder; the constituents and impurities sections, however, are not the same.

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Following rows concern ALL iron powders including Carbonyl Iron Powder |  |  |
| Substance composition | Heading only | Create a block here for each registered form of iron |
| Name | Iron Powders | Enter the right name according to the registered substance(s) |
|  | Carbonyl Iron Powder |  |
| Brief description | Iron Powders refer to iron powders made by sponge iron process, atomization processes (gas and water) and electrolysis | Enter the right description according to the registered substance(s) |
|  | Carbonyl Iron Powder refers to Iron Powder uncoated, made by thermal decomposition of iron pentacarbonyl. |  |
| Composition ID |  | A number is automatically created here by IUCLID. |
| Degree of purity flags: | Heading only | Click on the flag if you want to assign confidentiality and programme restriction |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Degree of purity | > 80% [w/w] | This is the same for all iron powders. |
| Following rows concern iron powders EXCEPT Carbonyl Iron Powder |  |  |
| Constituents | Heading only |  |
| Reference substance | iron / iron / iron / 7439-89-6 | To locate the reference substance from the IUCLID data base, click on this icon [see red arrow in screenshot above]. Select your substance from the database by typing in the name, EC or CAS number, click SEARCH, select the substance name and click Assign [see screenshot below]. Two problems may arise: • If no entry is found, you have first to import the substance from the EC inventory to the reference substance inventory. • If an entry is found but inactive, right mouse click and set to “active reference substance.” In order to simplify matters, the Iron Platform will provide reference substance files which member registrants can import into their IUCLID dossiers. |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 97 ≤ 100 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Impurities | Heading only | Create a block for each impurity. To locate the impurity substance from the IUCLID data base, click on this icon [as for section 1.1 above]. Select your impurity substance from the database by typing in the name, EC or CAS number, click SEARCH, select the substance name and click Assign. Two problems may arise: • If no entry is found, you have first to import the substance from the EC inventory to the reference substance inventory. • If an entry is found but inactive, right mouse click and set to “active reference substance.” In order to simplify matters, the Iron Platform will provide reference substance files for the impurities below which you can import into your IUCLID dossier. |
| Reference substance | chromium / Chromium / Chromium / 7440-47-3 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 0 ≤ 0.2 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | copper / Copper / Copper / 7440-50-8 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 0 ≤ 0.2 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | phosphorus / Phosphorus / Phosphorus / 7723-14-0 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 0 ≤ 0.2 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | silicon / Silicon / Silicon / 7440-21-3 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 0 ≤ 0.2 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks | Leave blank | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | manganese / Manganese / Manganese / 7439-96-5 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 0 ≤ 0.22 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | carbon / Carbon / Carbon / 7440-44-0 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 0 ≤ 0.2 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | oxygen / Oxygen / Oxygen / 7782-44-7 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | > 0 ≤ 1.5 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Following rows concern ONLY Carbonyl Iron Powder. |  |  |
| Constituents | Heading only | Create a new block |
| Reference substance | iron / iron / iron / 7439-89-6 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range | ≥ 97.8 ≤ 99.8 % (w/w) | This is per the sameness specification included in the Chemical Safety Report for Iron. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Impurities | Heading only | Create a new block |
| Reference substance | phosphorus / Phosphorus / Phosphorus / 7723-14-0 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range |  | Leave blank as not specified. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | carbon / carbon / carbon / 7440-44-0 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range |  | Leave blank as not specified. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | nitrogen / Nitrogen / Nitrogen / 7727-37-9 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range |  | Leave blank as not specified. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Impurities | Heading only | Create a new block |
| Reference substance | oxygen / Oxygen / Oxygen / 7782-44-7 |  |
| Typical concentration |  | Enter the concentration level or range for your substance here. |
| Concentration range |  | Leave blank as not specified. |
| Remarks |  | Enter any comments that you may wish to make with respect to the concentration level or range of your substance here. |
| Do not tick : “this impurity is considered relevant for the classification and labelling of the substance” |  |  |
| Following row concerns ALL iron powders including Carbonyl Iron Powder. |  |  |
| Additives | Heading only | Do not create a block here |

## 1.3	IDENTIFIERS

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Regulatory programme identifiers | Heading only | Click on |
| Flag | Heading only | Click on the flag if you want to assign confidentiality and programme restrictions. |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Regulatory programme |  | Select REACH Pre-registration number or REACH Inquiry number from the pick list. |
| ID |  | Enter your pre-registration or inquiry number. |
| Remarks |  | Leave blank |
| Other IT system identifiers | Heading only | Leave blank |

## 1.4	ANALYTICAL INFORMATION

For guidance on the analytical methods to use, please refer to the ANALYSIS METHODS FOR USE IN DEMONSTRATING SAMENESS - IRON document in the guidance and training for REACH registrants on the Iron Platform website or via the link in this sentence.

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Analytical information flags: |  | Click on the flag if you want to assign confidentiality and programme restriction |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Analytical methods and spectral data |  | Attach a document describing the analytical methods used. The name of the file will automatically appear in this field. |
| Optical activity |  | Leave blank |
| Results of analysis - chemical analysis |  | Heading only |
| Analysis type | Chemical analysis to prove sameness |  |
| Tested substance |  | Identify the substance tested, e.g. form of iron powder..... |
| Analysis results |  | Attach a file with your analysis results. The name of the file will automatically appear in this field. |
| Method used |  | Specify the analysis method and standard in accordance with which the analysis has been carried out, e.g. XRF in accordance with ISO ... |
| Remarks |  | If you feel the need to provide an explanation on the results of the analysis, include it here. |
| Results of analysis |  | If you carried out this [or any other] test, you will need to create a new block for this section and then fill in the fields as for the preceding sections. |

## 1.5	JOINT SUBMISSION

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Joint submission | Heading only | Create a block |
| Joint submission flags: |  | Click on the flag if you want to assign confidentiality and programme restriction |
| confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| programme restrictions |  | Select EU: REACH from pick list. |
| General information |  |  |
| Joint submission name | Iron (Einecs number 231-096-4) |  |
| Remarks |  | Leave blank |
| Regulatory programme |  | Leave blank or select EU: REACH |
| Lead | Heading only | Leave blank |
| Members | Heading only | Leave blank |

## 1.6	SPONSORS

Registrants may specify different Sponsor organisations, e.g. a Competent Authority in the context of the OECD HPV Chemicals programme or a Company in the context of the US EPA HPV Challenge programme.  Otherwise leave this section blank.

## 1.7	SUPPLIERS

Leave this section blank unless you are Only Representative. Although not mandatory, ECHA recommends that as an Only Representative you should attach clear documentation of your appointment as Only Representative, for example a copy of the appointment letter sent to importers.   In this case you are also advised to indicate the list of importers’ names covered by the registration in the field “Other importers”.  The Iron Platform strongly advises Only Representatives to follow ECHA’s recommendation.

## 1.8	RECIPIENTS

A recipient can be a Downstream user, a Distributor or a Customer (e.g. in the context of Product and process orientated research and development (PPORD)) being supplied with a Substance, or a Mixture or an Article. These definitions never include consumers.

# 3. MANUFACTURE, USE AND EXPOSURE

## 3.1	TECHNOLOGICAL PROCESS

Create a block for each process, i.e. each registered form of iron

Note: the related manufacture is linked to the manufacture part of the section 3.5 – Life Cycle description. This field is not subject to TCC rules until 2014 (see ECHA Q&A on IUCLID 5.4 (April 2012) for more information). Members are free to fill in this field.

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Methods of manufacture of substance | Heading only | Click on to add a block |
| Technological process flags: | Heading only | Click on the flag if you want to assign confidentiality and programme restrictions. |
| Technological process flags: |  | Click on the flag if you want to assign confidentiality and programme restriction |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Methods of manufacture | Iron powder via atomization process (gas and water) A thin stream of molten metal is disintegrated by the impact of high-pressure jets of liquid water or gas (air, nitrogen, argon). Tiny droplets of iron are formed and rapidly solidify into small particles of iron metal. With high-pressure water jets, the shape of the iron particles is irregular. Iron granules are collected, dried and ground to the set size distribution. The iron powder is then reduced in a decarburizing furnace to achieve specific chemical composition and desired metallurgical properties. The powder is finally milled and homogenised. Additives are eventually added before the packaging and the shipping to the customers Sponge iron powder The Höganäs process is a direct reduction method for production of sponge iron powder using magnetite ore (Fe3O4) and carbon as raw materials. The reduction process takes place in a gas fired tunnel kiln at around 1200°C for 60 hours producing a sponge iron cake that is crushed, milled and magnetically separated from impurities. The final reduction is made in a continuous furnace under hydrogen atmosphere. Before the packaging, particles are screened to achieve the desired particle size Iron powder via electrolysis The manufacture of iron powder by electrolysis is a batch process. The anodes are usually made of iron or low carbon steel. The electrolyte of the cell consists of a sulphate solution (temperature 50°C). Pure iron is deposited as a brittle lightly adhering sheet on the cathodes. After washing, drying, and stripping, the deposited iron is ground. Further milling and annealing is carried out to meet requirements of specific applications. | If you are an importer or Only Representative enter as the case may be “Importer/Only Representative - not relevant.” EU manufacturers should create a block for each process, i.e. each registered form of iron. This is the text submitted by the Lead Registrant - we suggest that EU manufacturers copy it unless it does not cover the method of manufacture, in which case amend or re-draft as appropriate. |
| Related manufactures | Heading only | Add a block by clicking on |
| Related manufacture |  | Click on to select the item related to iron powders |
| Methods of manufacture | Carbonyl Iron Powder - Iron Powder uncoated, made by thermal decomposition of Iron pentacarbonyl Carbon monoxide reacts under pressure with iron powder to form iron pentacarbonyl, Fe(CO)5. Iron pentacarbonyl is purified by distillation (residual metals typically < 10 ppm). Purified iron pentacarbonyl is vaporized by heating and decomposed to Carbonyl iron powder and CO.. During the decomposition process, iron particles grow on nuclei to form spherical iron particles. This process achieves properties (chemical composition, particle shape and distribution) unmatched by water-atomization or gas-atomization processes. The typical size range of Carbonyl Iron Powder is from < 1 to 8 microns (primary particles) . Raw iron powder grades can be sent to finishing processes such as milling, classifying, mixing and coating according to further uses. |  |
| Related manufactures | Heading only | Add a block by clicking on |
| Related manufacture |  | Click on to select the item related to carbonyl iron powder |
| Methods of article production | Heading only | This field has to be completed only when there is possible exposure from an article containing a substance intended for release. This field is not subject to TCC rules until 2014. Members are free to describe production of their own articles production in case they have concerns. |

## 3.2	ESTIMATED QUANTITIES

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Year |  | Enter the current year |
| Estimated quantities flags: |  | Click on the flag if you want to assign confidentiality and programme restriction |
| confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| programme restrictions |  | Select EU: REACH from pick list. |
| Total tonnage |  | If the substance has been imported or manufactured for at least three consecutive years, the tonnes per year shall be calculated on the basis of the average tonnes manufactured or imported in the three preceding calendar years. If the substance has not been manufactured or imported for three consecutive years then the tonnes manufactured or imported in a calendar year should be used (see ECHA guidance on registration, May 2012) |
| Details on tonnages |  | If you feel the need to provide an explanation for the basis of your tonnage, include it here. |

## 3.3	SITES

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Sites | Heading only | Create a block by clicking on |
| Site |  | Click on to select an existing site or create a new site Enter the name and location of your site(s). The minimum contact address information is town/city and country, but ECHA recommends filling all address fields. An Only Representative or Importer can assign a site, but this is not mandatory. If “Manufacturer” is selected in section 1.1, at least one production site must be entered in section 3.3. |
| Site flags: |  | Click on the flag if you want to assign confidentiality and programme restriction |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Legal entity owner |  | Assign the name of the legal entity which owns the site from the drop down list. |
| Manufacture/own use(s) |  | Create a block. Click on . Select appropriate manufacture(s) or use(s) from the section 3.5 Life Cycle description. This field is not subject to TCC rules until 2014. Members are free to fill in this field. |

## 3.4	INFORMATION ON MIXTURES

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Information on mixtures | Heading only | Add block(s) if necessary. |
| Flags |  | Click on the flag if you want to assign confidentiality and programme restrictions |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Trade name of mixture |  | Enter the trade name of your mixture. |
| Type of mixture |  | Enter a description of your mixture: e.g. powder |
| Typical concentration |  | Insert the typical range of the concentration of iron in the mixture. Ensure that the figure given for iron is consistent with the value[s] for other constituents given in other registration dossiers for this mixture. |

## 3.5	LIFE CYCLE DESCRIPTION

Information on uses is available on the Iron Platform website in the document Identified uses of iron the iron SIEF documents area or via the link in this sentence. Table 1 refers to Iron Powders (Iron Powder made by sponge iron process, atomization processes (gas and water) and electrolysis) and table 2 to Carbonyl Iron Powders (Iron Powder uncoated, made by thermal decomposition of Iron pentacarbonyl).

There is conflicting advice as to whether member registrants should select only certain uses or should select them all - the advice of the Iron Platform is to select them all so as not to have to modify the dossier in the future when a new use is entered into.

The Iron Platform provides an IUCLID file containing all the uses for Iron which you can upload to your IUCLID file and then copy/paste into the corresponding fields of your substance file (IUCLID files for download) (link to i5z files here)

If you wish to specify only certain uses, create a block here by clicking on the “Add” button and tick the relevant options.

Note: New fields were added during the IUCLID upgrade to version 5.4.0. (See fields with the explanation ‘New field, you are free to fill it” in the table below). These fields are not subject to TCC rules until 2014 (see ECHA Q&A on IUCLID 5.4 (April 2012) for more information). However, member registrants are free to fill them in if they wish.

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Life Cycle description | Heading only |  |
| Generic information | Heading only |  |
| Justification why no identified uses are reported |  | Leave blank - unless you have not added uses below in which case select in the pick list the justification about the absence of reported uses |
| Manufacture | Heading only |  |
| Manufacture name |  |  |
| Tonnage of the substance |  | New field, you are free to fill it |
| Number of sites range |  | New field, you are free to fill it |
| Environmental Release Category |  | Select relevant options in the picklist |
| Process Category |  |  |
| Use coverage in CSR |  | New field, you are free to fill it |
| Formulation | Heading only |  |
| Identified use name |  |  |
| Brief description of use process |  |  |
| Tonnage of the substance |  | New field, you are free to fill it |
| Number of sites range |  | New field, you are free to fill it |
| Environmental Release Category |  | Select relevant options in the picklist |
| Process Category |  |  |
| Product Category formulated |  |  |
| Technical function of the substance during formulation |  |  |
| Substance supplied to that use in form of |  |  |
| Use coverage in CSR |  | New field, you are free to fill it |
| Use at industrial site |  |  |
| Identified use name |  |  |
| Brief description of use process |  | New field, you are free to fill it |
| Tonnage of substance |  | New field, you are free to fill it |
| Number of sites range |  | New field, you are free to fill it |
| Environmental Release Category |  | Select relevant options in the picklist |
| Process Category |  |  |
| Product Category used |  |  |
| Sector of end use |  |  |
| Technical function of the substance |  |  |
| Substance supplied to that use in the form of |  |  |
| Subsequent service life relevant for that use |  |  |
| Link to subsequent service life |  | New field, you are free to fill it |
| Use coverage in CSR |  | New field, you are free to fill it |
| Consumer Uses | Heading only |  |
| Article service life | Heading only |  |
| Service life name |  |  |
| Tonnage of substance |  | New field, you are free to fill it |
| Article used by |  |  |
| Article category related to subsequent service life |  |  |
| Further description of article |  | New field, you are free to fill it |
| Exposure related description of article |  | New field, you are free to fill it |
| Environmental Release Category |  | Select relevant options in the picklist |
| Process Category for articles used by workers |  |  |
| Typical concentration of the substance in article % |  | New field, you are free to fill it |
| Technical function of the substance |  | Select relevant options in the picklist |
| Use coverage in CSR |  | New field, you are free to fill it |

## 3.6	USES ADVISED AGAINST

Do not create a block for Section 3.6 and leave it blank as there are no uses advised against.

## 3.7	Exposure Scenarios, exposure and risk assessment

The former section 3.7 Waste from production and use (IUCLID 5.3 and former versions) is attached as an htlm file in the section 3. Delete it if you need to update and resubmit your dossier since html attachments are not supported by ECHA business rules checks.

The former section “3.8 Exposure estimates” was replaced by the following subparts.

### 3.7.1	Exposure scenarios and local assessment

Note: This part was added in June 2012 during the upgrade to IUCLID 5.4.0. It will be taken into account by the TCC in 2014.

No endpoint should be created; the substance is not classified for environmental and human health endpoints, hence no exposure scenario should be developed.

### 3.7.2	Environmental assessment for aggregated sources

Note: This part has been added in June 2012 during the upgrade to IUCLID 5.4.0. It will be taken into account by the TCC in 2014.

This section should be left blank since the substance is not classified for environmental and human health endpoints.

### 3.7.3	Generic exposure potential

Note: this section was previously part of the former 3.5 Identified uses.

| ITEM | TEXT TO BE ADDED | EXPLANATION |
| --- | --- | --- |
| Flags |  | Click on the flag if you want to assign confidentiality and programme restrictions |
| Confidentiality |  | Leave blank or select the right level of confidentiality. If confidentiality is required, a justification has to be provided. |
| Programme restrictions |  | Select EU: REACH from pick list. |
| Closed system |  | This box should be ticked when the substance is used in a closed system, such as the use of liquids in hydraulic systems, cooling liquids in refrigerators and lubricants in engines and dielectric fluids in electric transformers and oil in heat exchangers. |
| Significant routes of exposure |  | Either tick the same boxes as the Lead Registrant or make your own selection as appropriate. The Lead Registrant has ticked the following boxes: Human exposure: - Oral - Dermal - By inhalation Environmental exposure: - Water - Air - Solid waste - Soil Pattern of exposure: - Continuous / frequent |

## 3.8	BIOCIDAL INFORMATION

Section 3.9 should be left blank as it is not relevant in this case - do not create a block for it.

# RECOMMENDATIONS CONCERNING SUBMISSION

Before submitting your dossier, do not forget to:

- check your substance file with the TCC Tool plug-in:

- If TCC fails, correct all mistakes or create a new substance file

- If TCC passes, go to the next step

- create a dossier by right clicking on your substance

- check your dossier file

- If TCC fails, create a new dossier file

- If TCC passes, go to the next step

- export your dossier file on your computer by right clicking on the dossier

- open your account on ECHA REACH-IT

- if the dossier file size is larger than 20MB => request a large file access code before submission on ECHA REACH-IT [it is normally immediate and you will receive the code in your REACH-IT message box]

- follow the prompts to submit your dossier file [for more detailed information, please consult the ECHA Guidance on submission (July 2012)]

After submission:

- check your message box in ECHA REACH-IT to follow progress of ECHA’s 14 dossier examination steps via the submission report;

- take the necessary actions, for example paying the registration fee.

Disclaimer: The Iron Platform and Iron Platform Services Ltd. do not make any representations or warranties in relation to the content of this guidance document. In particular, the Iron Platform and Iron Platform Services Ltd. do not make any representations or warranties regarding the accuracy, timeliness or completeness of its content. The Iron Platform and Iron Platform Services Ltd. will not be responsible for any loss or damage caused by relying on the content contained in this document.

Guidance version

| Date | Version - Main changes |
| --- | --- |
| October 30th, 2012 | Minor changes |
| August 13th, 2012 | Changes to fit the new IUCLID version (5.4.0) |
| September 8th, 2010 | Minor changes |
| August 26th, 2010 | First version |